ZEISS Axioscan 7

The ZEISS Axioscan 7 is an automated slide scanner designed for high-throughput imaging of biological samples. It supports brightfield, TIE contrast, and fluorescence imaging, enabling fast and reproducible digitization of slides for research and pathology workflows.

Features:

| Imaging Modes | Brightfield and fluorescence imaging |
| --- | --- |
| Stage | Motorized slide magazine with up to 100 slides |
| Detector | Axiocam 705 color camera Axiocam 712 monochrome camera |

Objective lenses:

| Magnification | Immersion | Type | Numerical Aperture | Working Distance (mm) |
| --- | --- | --- | --- | --- |
| 5X | Air | Fluar | 0.25 | 12.5 |
| 10X | Air | PlanApo | 0.45 | 2.0 |
| 20X | Air | PlanApo | 0.80 |  |
| 20x | Air | Plan-Neofluar | 0.50 | 2.0 |
| 40x | Air | PlanApo | 0.95 |  |

Laser module:

- LED light source (Viluma 7)

Multiple filter sets (91 HE LED, 112 HE LED), with dedicated bandpass filter

| Description | Excitation (nm) |
| --- | --- |
| DAPI | 405 |
| FITC | 488 |
| TRIC | 561 |
| Cy5 | 640 |
| Cy7 | 750 |

Software:

https://www.zeiss.com/microscopy/en/products/software/zeiss-zen-lite.html

Download Zen installer and follow the steps to install Zen Lite